THE CHLOR-ALKALI INDUSTRY

Chlorine is manufactured by the electrolysis of brine (saturated sodium chloride solution).

The overall reaction is:

2NaCℓ (aq) + 2H2O(ℓ) → Cℓ 2(g) + H2(g) + 2NaOH(aq)

There are three different cells:

- Mercury cell

- Diaphragm cell

- Membrane cell

The mercury cell uses the least energy of the three cells but it has the lowest yield of chlorine.  Mercury is also harmful to the environment.

The diaphragm cell produces a dilute sodium hydroxide solution and it uses an asbestos diaphragm, which is also harmful to the environment.

The membrane cell:  LEARN WELL!!!

The membrane used is a polymer, which is quite expensive to make but it has no harmful environmental issues.  This method produces very pure sodium hydroxide.

NITROGEN GAS

Nitrogen occurs naturally in the atmosphere. It forms 78% of the air. Pure nitrogen is obtained by from the fractional distillation of liquid air.

HYDROGEN GAS

Hydrogen gas is produced at Sasol from steam and coal by the process of gasification. These are heated together with oxygen at very high temperatures and pressure.

## Aluminium Extraction

Aluminium is mine as an ore called Bauxite which is washed by NaOH to produce alumina (Al2O3) and mostly Fe2O3 which is known as read mud.  Then aluminium has to be separated from oxygen during extraction. The bauxite has to be made molten before it can start conducting electricity. However, Al2O3 melts at 2050◦C which requires a lot of energy that is generated from electricity. These temperatures consume lot of energy and that may lead to less profit being obtained. Therefore, alumina is dissolved is cryolite (Na3AlF6) which reduces the melting point temperature of alumina to 900 – 1000 ◦C. This invention is cost effective because the amount of electricity used is reduced by more than half. Also this help in reduction of CO2 on electricity production of which it’s also an environmental saving measure. Since CO2 is a greenhouse gas. The aluminium extraction cell uses carbon electrodes (both anode and Cathode). After alumina has been electrolyzed it produces Al3+ and O2-; of which Al3+ is attracted by a negative electrode which is a cathode and it undergoes reduction there. Whereas O2- is attracted by a positive electrode which is an anode and it get oxidized there. The reactions occurring in this cell are as follows:

Anode: 	2O2- 	→ O2  + 4e-

Cathode: 	Al3+ + 3e-  → Al

Nett:		4Al3+ + 6O2- → 4Al + 3O2

However the production of O2 at the Carbon anode may lead to a side reaction which reduces the size of this electrode.

That side reaction C + O2   →  CO2. This CO2 re-joins the environment. Therefore, after sometime anode may need to be replaced.

The diagram below shows the simple representation of aluminium extraction cell.

Pure Al

Pure Al

SUMMARY OF ELECTROLYTIC CELL VS VOLTAIC (GALVANIC) CELL

|  | ELECTROLYTIC | VOLTAIC (GALVANIC) |
| --- | --- | --- |
| Appearance of simple form of cell | - Must have cell or battery in external circuit to supply electrical energy. - Consists of 2 electrodes in the SAME solution. | V KNO3 (aq) salt bridge electrons electrons Anode (-) Cathode (+) Ions V KNO3 (aq) salt bridge electrons electrons Anode (-) Cathode (+) Ions - No external source of electricity i.e. there will be no cell or battery shown in the external circuit. - It consists of two half cells containing different electrodes each of which is in a solution of its salt. The solutions in each half cell are DIFFERENT. - There must either be a salt bridge or some sort of porous membrane separating the two half cell to allow for the passage of ions between cells. |
| Energy conversion | Electrical to chemical | Chemical to electrical |
| Polarity of electrodes | Anode = positive Cathode = negative | Anode = negative Cathode = positive |
| Uses | Extraction of aluminium; purification of copper; electroplating; chlor-alkali process – electrolysis of brine to form chlorine, sodium hydroxide and hydrogen | Batteries - Primary – zinc-carbon; lithium; mercury (not rechargeable) - Secondary – lead-acid accumulator or car battery (rechargeable) |

QUESTION 1: DBE Exemplar 2014

The voltaic cell represented below functions at standard conditions.

V

graphite

graphite

I2 (s)

H+ (aq)

MnO4- (aq), Mn2+ (aq)

I- (aq)

V

graphite

graphite

I2 (s)

H+ (aq)

MnO4- (aq), Mn2+ (aq)

I- (aq)

1.1	Write down the concentration of H+ (aq) in the one half-cell.		    	(1)

1.2	Solids present in half-cells are usually used as electrodes.  Give a reason why

I2(s) is not suitable to be used as an electrode.				    	(1)

1.3	Write down TWO properties of graphite, other than being a solid, that makes it

suitable for use as electrodes in the above voltaic cell.		    	    	(2)

1.4	For the above voltaic cell, write down the :

1.4.1	NAME of the oxidizing agent.						    			(1)

1.4.2	Net cell reaction							    	    			(3)

1.4.3	Cell notation								    	    			(3)

1.5	Calculate the cell potential of the above cell.				    		(4)

1.6	How will the reading on the voltmeter be affected if the concentration of MnO4-

decreases?  Only write down INCREASES, DECREASES or NO EFFECT. (1)